Product Specification

27 53 13 Wireless Clock System & 27 51 23.50 Educational Intercommunications and Program Systems

Primex OneVue™ Sync 72MHz Wireless Clock System with Notify Bell Scheduling

Last update: 7/28/2026

Specifier Note: This product specification is written according to the Construction Specifications Institute (CSI), MasterFormatTM, SectionFormat, and PageFormat, contained in the CSI Manual of Practice. The section must be carefully reviewed and edited by the Architect/Engineer/Consultant to meet the requirements of the project and local building code. Coordinate this section with other specification sections and the drawings. Delete all “Specifier Notes” when editing this specification.

CONTENTS

27 53 13 Wireless Clock System & 27 51 23.50 Educational Intercommunications and Program Systems	1

PART 1 GENERAL	3

1.1 GENERAL REQUIREMENTS & SCOPE	3

1.2 RELATED SECTIONS	3

1.3 REFERENCES	3

1.4 DEFINITIONS	4

1.5 SYSTEM DESCRIPTION	5

1.6 REGULATORY REQUIREMENTS	7

1.7 SUBMITTALS	8

1.8 SUBSTITUTIONS	8

1.9 QUALITY ASSURANCE	9

1.10 DELIVERY STORAGE AND HANDLING	9

1.11 PROJECT SITE CONDITIONS	9

1.12 WARRANTY	9

1.13 SOFTWARE MAINTENANCE	9

PART 2 PRODUCTS	11

2.1 SECTION INCLUDES	11

2.2 Manufacturer	11

2.3 SYSTEM OPERATION	11

2.4 EQUIPMENT	13

PART 3 EXECUTION	22

3.1 EXAMINATION	22

3.2 INSTALLATION	22

3.3 FIELD INSPECTION	22

3.4 Manufacturer SERVICES	22

3.5 CLEANING	23

3.6 DEMONSTRATION	23

3.7 PROTECTION	23

3.8 TESTING	23

PART 1 GENERAL

1.1 GENERAL REQUIREMENTS & SCOPE

The system is specified as described.

- Furnish and install a complete new Wireless Clock System with Bell Scheduling System using the Primex OneVue™ Sync 72MHz Wireless Clock System with Notify Bell Scheduling.

- Furnish and install all system devices, accessories, and material in accordance with these specifications and drawing to provide a complete and operating system.

- All bids shall be based on the equipment as specified herein. The model designations are that of Primex. Inc. The specifying authority must approve any alternate system

Specifier Note: Edit the following system devices as required for the project.

- System shall include the System Devices

Event Bell Controller

Transmitter (master)

Repeater Transmitter (secondary)

GPS Receiver

Analog Clocks

Digital Clocks

Digital Elapsed Timers

Digital Code Blue Timers

1.2 RELATED SECTIONS

Specifier Note: Edit the following list as required for the project. List other sections with work directly related to this section.

- Division 26 “Electrical”

- Division 26 Section “Common Work Results for Electrical”

- Division 27 Section "Common Work Results for Communications"

- Division 27 Section "Communications Horizontal Cabling"

1.3 REFERENCES

System devices specified shall meet or exceed the requirements of the following.

Specifier Note: List standards referenced in this section, complete with designations and titles. This article does not require compliance with standards, but is merely a list of those used.

- National Fire Protection Association (NFPA): 1. NFPA 70 - National Electrical Code (NEC).

- Manufacturer Installation and User Guides.

1.4 DEFINITIONS

This section provides commonly used terms within this specification.

- SaaS: Software-as-a-Service

- AWS: Amazon Web Services

- TLS: Transport Layer Security

- HTTPS: Hypertext Transfer Protocol Secure

- SMA: Software Maintenance Agreement

- GPS: Global Positioning System, a worldwide system that employs a constellation of satellites in an integrated network to determine geographic location anywhere in the world, and which employs and transmits Universal Coordinated Time, the world’s most accurate and reliable time.

- NTP: Network Time Protocol, used for synchronizing the clocks on computer networks and devices from either a public server or a separate server on a private local area network.

- UTC: Universal Coordinated Time

1.5 SYSTEM DESCRIPTION

Specifier Note: System description is based on the Primex OneVue™ Sync 72MHz Wireless Clock System with Notify Bell Scheduling.

Wireless Clock System General Specifications

- System shall operate on a 72MHz frequency. The 72MHz frequency efficiently transmits its signal through common building materials and across longer distances with less interference - ensuring all system devices receive system data updates.

- System transmitter shall require a FCC license for operation, resulting in a safe and interference free operation.

- Manufacturer shall offer system transmitters with a variety of power output levels to provide coverage for a single building or an entire campus.

- System shall wirelessly synchronize clocks and/or timers, and shall be capable of clock readouts in multiple time zones where desired.

- System shall provide a master time source that is transmitted wirelessly from the system master transmitter to secondary system devices. The master time source can either from a system supplied GPS Receiver or from an OWNER defined internal or external NTP server that the master transmitter can access via the OWNER'S Local Area Network (LAN). Up to three designated NTP Servers may be specified to ensure continuity of time synchronization. The master time will be synchronized to UTC.

- Hard wiring of system clocks for data communication shall not be required.

- System clocks shall automatically adjust for Daylight Saving Time in locations where DST is observed.

- Each system clock and other components shall use both precise time and synchronized time.

- Digital clocks shall be synchronized to within 10 milliseconds every 10 minutes and the system shall have an internal oscillator that maintains plus or minus four seconds per day between synchronization, so that clock accuracy shall not exceed plus or minus 0.2 seconds.

- Analog Clocks shall be synchronized to within 10 milliseconds 6 times per day when operating clock strikes 2:01 AM, 6:01 AM, 10:01 AM, 2:01 PM, 6:01 PM, and 10:01 PM, and the system shall have an internal oscillator that maintains plus or minus one second per day between synchronization, so that clock accuracy shall not exceed plus or minus 0.2 seconds.

- System shall include an internal clock reference so that failure to detect the master time source shall not result in the system clocks failing to indicate time. Additionally, system transmitters shall have an internal battery backup of up to eight hours in the event of a power failure so that settings and the correct master time will be instantly recalled upon restoration of power.

- System shall incorporate a “fail-safe” design so that failure of any component shall not cause failure of the system. Upon restoration of power or repair of failed component, the system shall resume normal operation without the need to reset the system or any component thereof.

- If a system transmitter stops transmitting valid time signals due to power failure, the system analog clocks will continue to function as accurate quartz clocks until a valid time signal is decoded. If signal transmission is not restored for three consecutive days, a clock's second hand will “two-step” as a visual indicator that the signal has been lost. A system digital clock/timer that has not decoded a valid time signal for a three consecutive days will display a visual indicator by flashing its colons continuously until a valid time signal is received. Should clocks lose both power and signal, the clocks will not function.

- Clock locations shall be as indicated and clocks shall be fully portable, capable of being relocated at any time.

- System shall provide a mobile configurator app (available for both iOS and Android) that allows OWNER to on-board a new transmitter and manage transmitter settings.

- System transmitters shall be equipped with Bluetooth® low energy wireless technology that allows the devices to send and receive communication to and from the system mobile configurator app.

- System shall include access to the Manufacturer system cloud-based software that allows OWNER to manage and monitor system transmitters and receive remote support services. Optionally, the system transmitters can be set to stand-alone mode, which does not allow for remote support services. A system that requires stand-alone computer-based system software or server hardware for system management or monitoring does not meet the requirements for this specification.

- System transmitters shall require connection to OWNER'S existing wired Ethernet network (DHCP or Non-DHCP) to use NTP as its time source and to also send data to and download settings from the system software.

- System software and mobile configurator app shall allow for over-the-air (OTA) firmware updates to a system transmitters.

- System software shall log, store, and display all operating diagnostic errors received from a system transmitter.

- U.S. only: System must operate in accordance with a “Radio Station Authorization”, Form FCC 601 – LM, granted by the Federal Communications Commission (FCC). This license will be issued to and held by the end user.

- CANADA only: The system must operate in accordance with a “Technical Acceptance Certificate” issued under the authority of Industry Canada and the Ministry of Industry. This license will be granted to and held by the end user.

Bell Scheduling System General Specifications

Specifier Note: Provides the facility Owner with the ability to schedule their bells for the entire school year for every school in their district from one simple web-based interface. They can quickly and easily change the schedules when needed for a weather-related late start, an impromptu pep rally, or in case of emergency. The PrimexEVENT Bell Controller uses the power of their existing public address system to trigger standard or custom ring patterns throughout every school day.

- System shall be licensed and delivered via a Software-as-a-Service (SaaS) model and is built-on and powered by Amazon Web Services (AWS).

- System is licensed on a subscription basis centrally hosted by Manufacturer and delivered to Owner as a cloud-based software system with system devices.

- Manufacturer shall offer subscription pricing models, which are to be inclusive of software access, technical support, enhancements, and maintenance.

- System shall not require the installation of on-site system hardware or software, with the exception of the system devices that transmit data to and from the system software.

- System software revisions and system device firmware revisions are provided under the Manufacturer subscription agreement.

- System shall provide bell scheduling, system data storage, alerting, and reporting of the system devices, and user-defined system software settings.

- System shall maintain and store redundant, data backups for up to seven years.

- System can be scaled from a single building to a network of buildings, or an enterprise spread across many time zones.

- System shall allow for system device over-the-air firmware updates.

- System device firmware shall perform automated diagnostic monitoring of its battery life.

- System devices shall have the ability to store up to 7 day schedule for each relay; a total of 14 days of schedules.

- System devices shall have the ability to operate on battery backup power for up to a maximum of 24 hours.

- System software shall allow system users to create, edit, and manage bell schedules.

- System software shall allow zone schedules to be assigned to multiple bell controller devices.

- System software shall allow a bell schedule to include up to a maximum of 48 individual bell rings.

- System software shall allow scheduled periods to have specific ring types and allow system users to create customized ring types.

- System software shall allow for exception schedules that override a default schedule for a specified date range.

- System devices shall connect to system software each night to update the 7-day schedule(s) and hourly to update any individual schedule changes set in the system software.

- System devices shall transmit data to the system software over the OWNER'S existing Ethernet network.

- System software shall be accessed from a web browser and does not require the installation of software or other third-party plug-ins or applications.

- System users shall be able to access the system software from a web browser on a mobile or tablet device or computer with an internet connection.

- System software shall provide a dashboard view that displays the state of all bell controller devices and a list of system alerts and notifications.

- System software shall allow the OWNER'S system admin(s) to grant and control the data and features available to system users.

- System software shall allow system users to view current and historical data transmitted by system devices and the settings of system components. The data and settings viewable to a user shall be limited to the role(s) assigned to their system user profile, which is managed by OWNER'S system admin users.

- System software shall allow system admin user to set system wide user password complexity settings and session timeout settings.

- System software shall have the ability to generate alert messages to system users 24 hours a day, 365 days a year. System software shall allow system alerts to be acknowledged by system users within the system software or voice alert message. The system shall record and store the history of all alerting data and user actions.

Encryption and Authentication

- All user sessions between the web browser and the system software shall be encrypted by the HTTPS protocol.

- All network enabled system device communication over the OWNER'S network shall be secure and encrypted using the Transport Layer Security (TLS) encryption protocol and Secure Hypertext Transfer Protocol (HTTPS) authentication.

1.6 REGULATORY REQUIREMENTS

- Equipment and components furnished shall be of Manufacturer latest model.

- System shall be installed in compliance with local and state authorities having jurisdiction.

- U.S. only: The end user will hold a license, known as a “Radio Station Authorization” granted by the FCC. This license grants the end user protected use for wireless transmission at the designated frequency. This license will designate a unique “call sign” for each end user.

- U.S. only: Transmitter and receiver shall comply with Part 90 of FCC rules as follows: This device may not cause harmful interference. This device must accept interference received, including interference that may cause undesired operation. Transmitter frequency shall be governed by FCC Part 90.35. Transmitter output power shall be governed by FCC Part 90 257 (b).

- CANADA only: The end user will hold a license, known as a “Non Complex Fixed Station” Radio License granted by Industry Canada and the Ministry of Industry. This license grants the end user protected use for wireless transmission at the designated frequency.

- CANADA only: IC-2365: Application for “License to Install and Operate a Radio Station in Canada” must be completed and signed by end user prior to license issuance. The end user will grant permission for Primex to apply for the license on their behalf. Primex will provide all documents and technical information to Industry Canada for approval. This license will designate a unique “call sign” for each end user.

- CANADA only: Transmitter and receiver shall comply with RSS 119 of Issue 6 of Industry Canada specifications as follows: This device may not cause harmful interference, and this device must accept interference received, including interference that may cause undesired operation. Transmitter frequency shall be governed by IC: RSS119 Issue Transmitter output power shall be governed by IC: RSS119 Issue 6.

- Bell Scheduling System: Electrical Components, Devices, and Accessories: Listed and labeled per NFPA 70 by qualified testing agency.

- Bell Scheduling System: Regulatory Requirements: System design and installation shall comply with the following: National Electric Code (NEC).

Underwriters Laboratory (UL) standards

Local codes and regulations.

1.7 SUBMITTALS

- Product Data: Submit complete catalog data for each system device and components, describing physical characteristics and method of installation.

- Shop Drawings: Showing the following. 1. Diagram of proposed monitoring system showing SaaS platform, communication pathway, and schedule of individual system device installation locations. 2. Indicate integration with the OWNER'S network. Include a line diagram of network relationships. 3. Show system device power requirements. 4. OWNER'S existing public access (PA) system or bell system that is wired to system devices.

- Samples: Submit one specified system device model(s) for approval. Approved sample(s) shall be tagged and shall be installed at location directed.

US Specifier Note: In accordance with FCC regulations, an application for “Radio Station Authorization” must be filed prior to use of the equipment. Normally, the Manufacturer will have completed the filing and obtaining the license. If not, the Owner will be required to file the application with the FCC prior to use. Furnishing the license, or a copy of the application, will confirm that FCC approval has been obtained. CANADA Specifier Note: In accordance with Industry Canada regulations, an application for “Technical Acceptance Certificate” must be filed prior to use of the equipment. Normally, the Manufacturer will have completed the filing and obtaining the license. If not, the Owner will be required to file the application with the Industry Canada prior to use. Furnishing the license, or a copy of the application, will confirm that Industry Canada approval has been obtained.

- U.S. only: Operating License: Submit evidence of application for FCC Radio Station Authorization prior to installing equipment. Furnish the license or a copy of the application for the license, to the Owner/End User prior to operating the equipment. The original license must be delivered to the Owner/End User.

- CANADA only: Submit IC Technical Acceptance Certificate prior to installing equipment. Furnish the license or a copy of the application for the license, to the Owner/End User prior to operating the equipment. The original license must be delivered to the Owner/End User.

- Manufacturer Instructions: Submit complete installation, set-up and maintenance instructions.

Specifier Note: Informational submittals require review, but no response by A/E or Owner.

- Information submittal: Sample Warranty

- Information submittal: System Subscription Agreement

1.8 SUBSTITUTIONS

- Proposed substitutions, to be considered, shall be manufactured of equivalent materials that meet or exceed specified requirements of this Section.

- Proposed substitutions shall be identified not less than 10 days prior to bid date.

- Other systems requiring wiring and/or conduit between master and clocks and/or timers will not be accepted.

- Other systems using wireless technology in an unlicensed frequency range will not be accepted.

- Other systems using wireless technology where the license is held by any party other than the end user will not be accepted.

1.9 QUALITY ASSURANCE

- U.S. only: Permits: Operating license for the transmitter from the FCC.

- CANADA only: IC-2365: Application for “License to Install and Operate a Radio Station in Canada” must be completed and signed by end user prior to license issuance.

- Qualifications: Manufacturer: Company specializing in manufacturing commercial time system products with a minimum of 30 continuous years of documented experience including 10 or more years of experience producing GPS wireless time systems. Manufacturer of wireless and Ethernet connected system with a minimum of seven years record of satisfactory manufacturing and support of systems comparable to basis of specified system design.

- Installer: Company with documented experience in the installation of commercial time systems.

- Prior to installation a site survey must be performed to determine proper transmitter placement.

1.10 DELIVERY STORAGE AND HANDLING

- Deliver all components to the site in the Manufacturer original packaging.

- Packaging shall contain Manufacturer name and address, product identification number, and other related information.

- Store equipment in finished building and in unopened packaging until ready for installation.

1.11 PROJECT SITE CONDITIONS

- Clocks and/or Timers shall not be installed until painting and other finish work in each room is complete.

- Mobile configurator app: installed on a mobile device meeting the app install requirements.

- Transmitter with an external antenna: Coordinate installation of system antenna for access to the roof to comply with safety standards detailed in Manufacturer instructions and per local codes.

- GPS Receiver time source: Coordinate installation of device for access to the roof or exterior side wall per Manufacturer installation instructions.

- System design is integrated with OWNER'S existing network for use with system network devices.

- Bell Controller system device relay(s) are designed to be wired to OWNER'S existing public access (PA) or bell system to invoke the bell schedules managed in system software.

- Conductors and Cables: Comply with requirements of Division 27 Sections "Common Work

- Results for Communications" and "Communications Horizontal Cabling.

- Signal and Control Circuits: Manufacturer recommended stranded, single conductors, or twisted- pair cables.

- Data Circuits: Category 5 minimum, twisted-pair cable.

- Plenum Cable: Listed and labeled for plenum installation.

1.12 WARRANTY

- Manufacturer shall provide a 5 year limited warranty on system transmitters, clocks, digital timers, timer control switch, and GPS Receiver.

- All other system components shall have a 1 year limited warranty, including Event Bell Controller, external antennas and components and universal power supply (UPS).

1.13 SOFTWARE MAINTENANCE

Applies to the Bell Scheduling System.

- Software Maintenance Agreement (SMA): included in the subscription agreement for system.

- All updates, enhancements and maintenance are performed remotely by the Manufacturer per agreed upon SMA.

PART 2 PRODUCTS

2.1 SECTION INCLUDES

- The system and equipment is specified as described in this section.

- All bids shall be based on the equipment as specified herein. The model designations are that of Primex, Inc. The specifying authority must approve an alternate system.

2.2 Manufacturer

- System shall be manufactured by:

Primex, Inc., 965 Wells Street, Lake Geneva, WI 53147 | Phone: (800) 537-0464 | Fax: (262) 248-0061 | Email: info@primexwireless.com | www.primexwireless.com

2.3 SYSTEM OPERATION

The system shall perform in the sequence of operation as described below.

- Install system transmitters and components, including internal or external antenna and optional GPS Receiver, detailed in Manufacturer installation instructions.

- Configure system transmitter settings through mobile configurator app detailed in Manufacturer instructions.

- Configure and install system clock devices per model specifications detailed in Manufacturer installation instructions.

- Configure system bell controller and setup system software as detailed in Manufacturer instructions.

Master transmitter operation

Specifier Note: Select the system’s specified master time source:

- When power is first applied, it looks for a master time source (first GPS and then NTP). If a master time source is not found, it uses its onboard real time clock (RTC) and continues to search for its master time source. If time source is not found, transmitter is set to a warning state with a time sync failure status, its caution LED status indicator is illuminated, and transmits its state to the system software at its scheduled check-in time (default set to every 5 minutes). If it fails to get time from either source consecutively for 30 minutes (default), it enters an alarm state and its error LED indicator is illuminated. If the RTC clock is off significantly, the transmitter will report a critical error to OneVue. This will occur only after the first 30 minutes of operation if the RTC continues to be significantly off.

- GPS Time Source: With the transmitter in GPS mode, it powers to a connected GPS receiver mounted with a clear view of the sky. Upon power, the GPS receiver seeks the GPS satellites in orbit to determine position and UTC time. Once the transmitter acknowledges receivable GPS data, it downloads time data and synchronizes its internal master clock to GPS time. The transmitter then starts to transmit its internal time once every second. The transmitter updates its internal clock every time it receives valid time data from the GPS.

- NTP Time Source: Transmitter connects over the OWNER'S Ethernet to the IP address of the NTP server. This IP address is programmed into the transmitter as part of its setting configuration. Once the connection to the NTP server is acknowledged, it downloads time data and synchronizes its internal master clock to received NTP time. The transmitter then starts to transmit its internal time once every second. The transmitter updates its internal clock in this mode once every 5 minutes by system default (end-user setting). Transmitter configuration allows up to three NTP servers for fail-over purposes.

Clock operation

- After initial setup, the clock and/or timer shuts off its receiver. Six times each day an analog clock microprocessor activates its receiver and starting with the stored channel it search for a valid time signal. Every 10 minutes a digital clock/timer activates its receiver and starting with the stored channel search for a valid time signal. If necessary, the clocks will resynchronize to the correct time.

- If an analog clock has not decoded a valid time signal for three consecutive days, it enters step mode. Low battery voltage is a common cause of the clock to not properly decode a time signal. If a clock goes into step mode, replace its batteries first and then determine if the clock synchronizes to master time source before attempting other troubleshooting methods.

- If a digital clock/timer has not decoded a valid time signal for three consecutive days, its display colon indicator flashes continuously until a valid time signal is received.

2.4 EQUIPMENT

The system shall include all equipment as specified.

Supply following transmitter model(s)

- 1 Watt Transmitter with internal antenna, Ethernet Port (with GPS)

- 1 Watt Transmitter with external antenna, NTP and GPS Input

- 1 Watt Transmitter with internal antenna, NTP or Repeater capability

- 5 Watt Transmitter with external antenna, NTP and GPS input

- 30 Watt Transmitter with external antenna, NTP and GPS input

Specifier Note: Exclude external antenna section below if Transmitter(s) with an internal antenna is specified.

Supply following external antenna

- Non-penetrating Mast Antenna Kit

- Pipe to Pipe Mast Antenna Kit (for use with 5 and 30 Watt Tansmitters)

- Non-penetrating Mast Antenna Kit

1 Watt Transmitter specifications

Transmitter shall meet the specifications below:

- 1-Watt maximum transmission (at transmitter)

- 72MHz frequency range

- 49 channels available (pre-programmed prior to shipment)

- Dimensions: 17.0”L x 12.0”W x 1.7”D (43.2cm x 30.5cm x 4.32cm)

- Weight: 9 lb.. (4 kg)

- Power Supply: Input: 120 VAC, 50/60 Hz, 0.6 Amp | Output: 9 VDC, 2.78 Amp, 6 ft. (1.8 m) cord

- Operating Range: 32° to 122°F (0° to 50°C)

5 and 30 Watt Transmitter specifications

Transmitter shall meet the specifications below.

- Operating Frequency Range: 72MHz

- Enclosure Dimensions: 22”W x 17”H x 22”D (55.88cm x 43.18cm 55.88cm)

- Approx. Weight: 55 lb. (24.9 kg)

- Power Supply: 120 VAC, 50/60 Hz, 6.0A Max

- Oversized heat sink provides adequate cooling

- Amplifier Output Bandwidth: >20 MHz maximum

External antenna specifications

Specifier Note: Section only applies to EXTERNAL Antenna Transmitter models. Delete if a 1 Watt Transmitter with an Internal Antenna Transmitter model is specified.

Transmitter shall include an external antenna that meets the below specifications

- Antenna connects to the transmitter via a 100 ft. (30.5m) 50-ohm coaxial cable.

- 5 and 30 Watt models: External Antenna must be installed by Primex or a certified Primex installer.

- Cabling: Maximum 100 ft. (30.48 m) between transmitter and antenna

- FCC Part 90 Accepted

- IC RSS-119 Accepted

- Weight: 9 lb. (4.1 kg)

- Shall include an Outdoor Ground Plane Antenna that meets the specification below:

Dimensions: 29.4”H x 41.5”W (74.7cm x 105.41cm)

Lightning Protection: Direct Ground

Cabling: 100 ft. (30.48 m) between transmitter and antenna provided

FCC Part 90 Accepted

IC RSS-119 Accepted

Weight: 8.25 lb. (3.75 kg)

GPS Receiver specifications

Specifier Note: If using NTP only for time source input, you may delete this section.

- External Antenna model: GPS roof mounted with 16 ft. (5 m) cable attached. GPS Receiver shall be a complete GPS receiver including antenna in a waterproof case, designed for roof or outdoor mounting. Provide mounting bracket for attachment to roof structure.

- Internal Antenna model: GPS roof mounted, with 10 ft. cable (3 m) attached.

- An GPS extension cable is available from the Manufacturer, 50 ft. (15.25 m), 100 ft. (30.5 m), or 200 ft. (61 m). Use with an external antenna: total GPS cable length cannot exceed 100 ft. (30.5 m).

- GPS Receiver cable must be plenum rated where required by local code.

- GPS connection to main Transmitter requires ferrite bead connector (supplied with transmitter).

NTP or GPS Time Source

- 49 channel transmitter will allow for either NTP time input or GPS satellite time input with use of a GPS Receiver unit. Unit shall obtain current UTC time from either satellite via GPS or via NTP through an Ethernet port.

Repeater Transmitter

Specifier Note: Large buildings and multi-building projects may require repeater transmitter to provide proper coverage. Consult Primex for assistance in making this determination. If repeater transmitters are required, include this section.

- Shall receive signal from master Transmitter and transmit the signal to the system devices in its vicinity, which are out of range from the master transmitter.

Analog Clocks

Clocks shall meet the following specifications:

- Analog clocks shall be wall mounted.

- Face shall be white. Hour and minute hands shall be black.

- Additional colors, finishes, and dial faces are available from Manufacturer.

- Clock faces can be customized by Manufacturer to display organization name or logo as specified.

- Clock frames and lenses are of durable thermoplastic.

- Clocks shall have a tamper proof/theft resistant clock-lock mounting slots.

- Analog clocks shall be capable of automatically adjusting for Daylight Saving Time. An on-off switch located on the transmitter shall disable this function if desired.

- Clock shall have either a battery-power or a 120 VAC power supply built into the clock assembly.

- If power is interrupted, the clock will stop until power resumes. Upon resumption of power, the clock will self correct to the current time.

- Electric (AC) models will include a two prong plug. The power cord can be cut to length and sliced as needed for hard wired installation.

- Battery-operated analog clocks shall have up to a 5-year battery life. Battery life is based on common operating conditions and may very due to installed site conditions.

- Installer will furnish clock batteries in accordance with Manufacturer instructions.

- Time shall be automatically updated from the transmitter 6 times per day.

- If the transmitter stops transmitting valid time signals due to power failure, the clocks will continue to function as accurate quartz clocks until a valid time signal is decoded. If signal transmission is not restored after 96 hours, the second hand will “two -step” as a visual indicator that the signal has been lost. Should the clocks lose both power and signal, the clocks will not function.

- Analog clock receivers shall be as follows: Receiver sensitivity: >-110 dBm, Receiver power: Dual Alkaline batteries or AC-powered, Antenna type: internal, Antenna gain: -7 dBd

- SUPPLY MODELS - Traditional Series Analog Clocks

(Power Source) Description

(Battery) 9” (22.86cm) Black

(Battery) 12.5” (31.75cm) Black

(Battery) 12.5" (31.75cm) White

Battery) 16" (40.64cm) Black

(Electric) 12.5” (31.75cm) 120VAC Black

(Electric) 12.5” (31.75cm) 120VAC White

(Electric) 16” (40.64cm) 120VAC Black

(Battery) 12.5” (31.75cm) Black, Remote Antenna Clock

(Battery) 12.5” (31.75cm) White, Remote Antenna Clock

- SUPPLY MODELS - Slim Series Analog Clocks

Slim Metal Series Clock

(Power Source) Description

(Battery) 12.5" (31.8cm) Slim Mode

(Electric) 12.5" (31.8cm) Slim Mode

(Battery) 16” (40.6cm) Slim Model

(Electric) 16” (40.6cm) Slim Model

- SUPPLY MODELS - Gallery Series Analog Clocks

Analog Gallery Series

(Power Source) Description

(Battery) 24” (61cm) Distressed Brown Gallery Series Clock Roman Dial

(Battery) 24” (61cm) Distressed Brown Gallery Series Clock Arabic Dial

(Electric) 24” (61cm) Distressed Brown Gallery Series Clock Roman Dial

(Electric) 24” (61cm) Distressed Brown Gallery Series Clock Arabic Dial

- SUPPLY MODELS - Wood Series Analog Clocks

Wood Series Analog Clocks

Battery-Powered Clocks

Description

11.5” (29.21cm) Honey Arabic

11.5” (29.21cm) Honey Roman

11.5” (29.21cm) Dark Cherry Arabic

11.5” (29.21cm) Dark Cherry Roman

11.5” (29.21cm) Clear Oak Arabic

11.5” (29.21cm) Clear Oak Roman

11.5” (29.21cm) Walnut Arabic

11.5” (29.21cm) Walnut Roman

16” (40.64cm) Clear Oak Arabic

16” (40.6cm) Clear Oak Roman

16” (40.64cm) Dark Cherry Arabic

16” (40.6cm) Dark Cherry Roman

16” (40.6cm) Honey Roman

16” (40.6cm) Honey Arabic

16” (40.6cm) Walnut Roman

16” (40.6cm) Walnut Arabic

AC-Powered Clocks

Description

16” (40.64cm) Honey Arabic

16” (40.64cm) Honey Roman

16” (40.64cm) Dark Cherry Arabic

16” (40.64cm) Dark Cherry Roman

16” (40.64cm) Clear Oak Arabic

16” (40.64cm) Clear Oak Roman

16” (40.64cm) Walnut Arabic

16” (40.64cm) Walnut Roman

Digital Clocks

Clocks shall meet the following specifications.

- Digital Clock must be able to receive synchronized time signal from the master or satellite transmitter.

- Digital Clock display must include a 12 or 24-hour time display, a PM indicator light, and an alternating time and date display option.

- Surface-mount models are available from the Manufacturer.

- Flush-mount models are available from the Manufacturer.

- Dual-mount models are available from the Manufacturer; dual mount kit.

- Digital Clock shall be capable of automatically adjusting for Daylight Saving Time.

- Digital Clock shall have either a 120 VAC or 24 VAC power supply built into the clock assembly.

- 120 VAC Digital Timer will include a power cord with a two prong plug. Power cord can be trimmed to length and spliced as needed for hard wired installation.

- 24 VAC Digital timer will include a power cord with pigtail to be hard wired to building 24VAC power grid.

- Digital Clock must be viewable from 150 ft. (45.7 m).

- Digital Clock must have highly visible 7-segment LED digits.

- Digital Clock shall have three display dimmer options, 75%, 50%, and 25%.

- Digital Clock LED colors, sizes, digit layouts, and mount options are available from the Manufacturer.

- SUPPLY MODELS - Digital Clocks

Digital Clocks (Levo Series)

Surface-Mount with 2.5" Digits

Specifier Note: Indicate if 4-Digit or 6-Digit Model

Description

120VAC, 4° Slope Bracket, 10’ (3.0m) cord with plug

120VAC, 18° Slope Bracket, 10’ (3.0m) cord with plug

24VAC, 4° Slope Bracket, 12” (30.48cm) cord with pigtail

24VAC, 18° Slope Bracket, 12” (30.48cm) cord with pigtail

Surface Mount with 4" Digits

Specifier Note: Indicate if 4-Digit or 6-Digit Model

Description

120VAC, 4° Slope Bracket, 10’ (3.0m) cord with plug

24VAC, 4° Slope Bracket, 12” (30.48cm) cord with pigtail

Elapsed Timer

Timers shall meet the following specifications.

- Timer must also function as a clock or function as a count-down/count-up interval timer when programmed with a three button wall mount control switch.

- Timer must accurately count up or count down up to a maximum of 99 hours, 59 minutes, and 59 seconds.

- Timer shall include a three button wall mountable control switch. This control will be mounted in a single gang electrical box. Control cover plate shall be stainless steel. Control buttons must be washable with water and common disinfectants.

- Switch control shall connect to timer with a supplied reversed wire (cross-pinned) CAT5 cable with an RJ-45 connector or a reversed wire (cross-pinned) telephone cable with an RJ-11 connector. It can be extended up to 100 ft. (30.48 m).

- Timer switch control can be configured to simultaneously activate two connected timers.

- Timer shall be capable of automatically adjusting for Daylight Saving Time

- Timer must be viewable from 150 ft. (45.7 m).

- Timer must have highly visible 7-segment LED digits.

- Time shall have three display dimmer options, 75%, 50%, and 25%.

- Timer display shall include a 12 or 24-hour time display option, a PM indicator light, and an alternating time and date display option.

- Additional LED colors, sizes, digit layouts, mounts, and timer options are available from the Manufacturer.

- Timer must have an audible tone option on the count-up and count-time function with a frequency of 3KHz +/-0.5KHz.

- Timer shall have either a 120 VAC or 24 VAC power supply built into the clock assembly.

- SUPPLY MODELS - Elapsed Timers

Elapsed Timers

Surface-Mount Timers with 2.5" Digits

Description

120VAC, 4° Slope bracket, 10' (3.0m) cord with plug

24VAC, 4° Slope bracket, 12" (30.48cm) cord with pigtail

Flush-Mount Timer with 2.5" Digits

Description

120VAC, 30” (76cm) cord with pigtail

24 VAC, 12" (30.48cm) cord with pigtail

Surface-Mount Timers with 4" Digits

Description

120VAC, 4° Slope bracket, 10' (3.0m) cord with plug

24VAC, 4° Slope bracket, 12" (30.48cm) cord with pigtail

Code Blue Timer

Timers shall meet the following specifications.

- Timer must function as a clock and must integrate to OWNER'S existing code blue control system.

- Timer shall include a three button wall mountable control switch. The control switch will be mounted in a single gang electrical box.

- The timer switch control buttons must be washable with water and common disinfectants.

- The timer switch control shall connect to timer with a supplied reversed wire (cross-pinned) CAT5 cable with a RJ45 connector or a reversed wire (cross-pinned) telephone cable with an RJ-11 connector. It can be extended up to 100 ft. (30.48 m).

- Timer switch control can be configured to simultaneously activate two connected timers.

- Timer display must include a 12 or 24-hour time display, a PM indicator light, and an alternating time and date display option.

- Timer shall be capable of automatically adjusting for Daylight Saving Time.

- Timer must include an optional audible tone to alert changes in interval cycles in the count-up and count-down function.

- Timer must have adjustable brightness control.

- Timer must have power outage memory backup and maintain correct time for 8 hours without power.

- Timer shall have either a 120 VAC or 24 VAC power supply built into the clock assembly.

- SUPPLY MODELS - Code Blue Timer

Code Blue Timer - Levo Series

Timers with 2.5" Digits - 6-Digit

Description

120VAC, 4° Slope Bracket, 10’ (3.0m) cord with plug

120VAC, 18° Slope Bracket, 10’ (3.0m) cord with plug

24VAC, 4° Slope Bracket, 12" (30.48cm) cord with pigtail

24VAC, 18° Slope Bracket, 12" (30.48cm) cord with pigtail

Flush-Mount Timers with 2.5" Digits - 6-Digit

Description

120VAC, 30” (76cm) cord with pigtail

24 VAC, 12” (30.48cm) cord with pigtail

Timers with 4" Digits - 6-Digit

Description

120VAC, 4° Slope Bracket, 10’ (3.0m) cord with plug

24VAC, 4° Slope Bracket, 12" (30.48cm) cord with pigtail

Accessories

- System shall include the accessories below:

Analog Clock - Dual Clock Kit

Description

Black - Fits 12.5" Traditional Series or Educational Series Analog Clock

White - Fits 12.5" Traditional Series or Educational Series Analog Clock

Slim Metal Series Clock - Dual Clock Kit

Description

Fits 12.5” (31.75cm) Slim Model Clock

Fits 16” (40.6cm) Slim Model Clock

Digital Clock - Dual Clock Bracket Kit

Ceiling mount: [2", 4-Digit] [2", 6-Digit] [4", 4-Digit] [4", 6-Digit]

Wall mount: [2", 4-Digit ] [2", 6-Digit] [4", 4-Digit]

Digital Clock Polycarbonate Clock Guards

Description

2.5" (6.35cm) Guard: 16.0" L x 7.0" H x 5.5"D (40.64cm L x 17.78cm H x 13.97 cm D)

4" (10.16cm) Guard: 25.0" L x 9.8" H x 4.4" D (63.5cm L x 24.9cm H x 11.2 cm D)

Analog Clock Wire Clock Guard

Description

16" Clocks (40.64cm): 18" square x 3"D (45.72cm square x 7.6cm D)

Crash Cart Kit

Description

Complete kit for mounting a Digital Clock Timer to a mobile crash cart

Specifier Note: Where desired for mounting transmitter, specify the following "transmitter rack" (one for each transmitter).

1-Watt Transmitter Rack

  - 18 GA metal

  - Epoxy coated

  - 18”L x 3“H x 16.5“D (45.7cmL x 7.6cmH x 41.9cmD)

Description

1-Watt Transmitter Rack

Transmitter accessory pack - transmitter Rack and UPS

Event Bell Controller Device

Specifier Note: Event Bell Controller device is offered in a Ethernet/PoE model. Use with a wired Ethernet network requires AC power adapter. Battery-power is for backup power use only.

- Basis of Design Product: PrimexEVENT Bell Controller Device PoE (E130-E). Supplied with battery backup pack, and a 5 VDC (output) wall adapter AC power supply using a mini-USB connector.

- Ratings: 3 VDC Relay operation | NC & NO contacts are rated at: Resistive load: 8A at 250VAC / 30 VDC and Inductive Load: 3.5A at 250VAC / 30 VDC

- Enclosure: ABS plastic | Dimension: 4.7" H x 3.7" W x 1.3" D (11.93cm x 9.39cm x 3.30cm) | Weight: 0.3 lb. (136 gram) with 2 AA batteries

- Display: Liquid crystal display (LCD), dimension: 0.75 in. H x 1.38 in. W (1.90 cm x 3.50 cm) | LED Status Indicator: green, yellow, red

- Mounting: Keyhole slot with lock down screw holes in back panel for wall mount; or surface mount with supplied dual-lock adhesive mounting strips.

- Operation: Alerts: audio, visual LED, LCD display

- Configuration: configured from system software or locally at device with supplied device configuration software

- Stored Schedule Capacity: 7 day schedule for 2 zones

- Check-in button for manual download schedule request.

- Battery-power: For use as a backup power supply in the event of a AC or PoE power outage. 3.0v Primex Lithium/Iron Disulfide Battery Pack or two stand-alone 1.5v Lithium AA batteries. | Use of alkaline batteries is not recommended.

- Environment: Operating Temperature: 32 to 122° F (0 to 50° C), indoor use only | Storage Temperature: -4 to 140 ° F (-20 to 60° C)

- FCC, CE, and IC compliant

Event Bell Controller Accessories

Specifier Note: Only include those accessories specified.

- AC adapter: 5V DC USB Mini B (5 pin) connector interface | 2. 5 feet (1.5 m) cable | Input: 100-240 VAC, 50/60 Hz | Output: 5V, 1.0A max

- AC Adapter Extension: 6 feet (1.8 m)

- Battery Pack Replacement: 3.0v Lithium/Iron Disulfide Battery Pack (contains 2 AA 1.5V Energizer® Ultimate Lithium batteries)

PART 3 EXECUTION

3.1 EXAMINATION

- Examine conditions with the Installer present for compliance with requirements and other conditions affecting the performance of the system and the system devices.

- Do not proceed until unsatisfactory conditions have been corrected.

- Verify that construction is complete in spaces to receive equipment and that rooms are clean and dry.

- Verify that 120 VAC electrical outlet is located within 5 ft. (1.5 m) of device and the outlet is operational and properly grounded.

- Timer Switch Control: Verify single gang electrical box for switch control is mounted and within 15 ft. (4.5 m) of digital display device. Verify pathway for connecting cable is available and compliant to local building codes.

- PoE devices: Verify that an active PoE port is located within close proximity to device.

3.2 INSTALLATION

Specifier Note: To assure optimum performance of the system, a site survey is recommended to be performed by Primex or a Certified Primex installation company. To assure optimum performance of the Time Synchronization System, transmitter(s) location (s) must be specified in the construction documents. Contact Primex Technical Support at 1-800-404-8112.

- General: Install system devices in accordance with applicable codes.

- Install system devices in accordance with Manufacturer written instructions.

- Provide all system equipment necessary for a complete and operable system.

- System devices relays are to be wired to OWNER'S existing public access (PA) or bell system.

- Comply with requirements of Division 27 Sections "Common Work Results for Communications" and "Communications Horizontal Cabling."

- Cables: Install cables in raceways and cable trays except within consoles, cabinets, and desks [and except in accessible ceiling spaces and framed partitions where exposed wiring is allowed by Owner]. Install plenum cable where required. Conceal cable installation where possible.

3.3 FIELD INSPECTION

- Inspection: Make observations to verify that system devices and components are properly labeled.

- Prior to final acceptance, inspect each system device and component, adjust as required, and replace parts which are found defective.

- At completion of system device installation and prior to final acceptance, turn on the equipment; ensure that all equipment is operating properly, and that the system software and all system devices and components are functioning.

3.4 Manufacturer SERVICES

- System software and device manuals shall be assessable through Manufacturer online knowledge base

- Provide Manufacturer system commissioning in accordance with Manufacturer written recommendations. Perform operational testing to verify compliance with requirements. Adjust as required.

- Manufacturer to provide a specified level of system commissioning services.

Specifier Note: Only include one level of commissioning. Contact Primex for additional information.

- Phone commissioning: system deployment training, including system set up, validation of device configuration, and system functionality by way of a remote web conference.

- On-site commissioning: system deployment training, including system set up, validation of device configuration, system functionality, verification of device network connections, and device install training.

- Installation and full on-site commissioning: system training, software configuration, validation of device configuration, training on system functionality, verification of network use, device install training, and device installation.

3.5 CLEANING

- Prior to final acceptance, clean exposed surfaces of devices, using cleaning methods recommended by Manufacturer.

3.6 DEMONSTRATION

- Initial Demonstration and Training: provide demonstration and training for OWNER facility staff that is responsible to perform system software administration.

- Demonstrate maintenance procedures for system devices.

- Demonstrate the system software components, including the monitoring and management of alert rules, teams, reports, clocksbell controller devices, device gateways, device networks, and the account settings and system users.

3.7 PROTECTION

- Protect finished installation until final acceptance of the project.

3.8 TESTING

- All system devices must be tested at their operational installation location under normal operational conditions.